Lab Report-6

(Sanchit jalan,Group-3,2022101070,Table No:-40)

Experiment 5C:-

Objective:- To make 4 bit UP-DOWN counter.

Electronic components required:-

- Arduino UNO

- Digital test kit

- Wires

TinkerCAD reference circuit:-

Procedure:-

- Connect 4 pins of Arduino with 4 LEDS and connect power supply and ground of Arduino to breadboard.

- Write the appropriate code for programming Arduino.

- Most of the code was provided , we just had to program the code to make it oscillate from 0 to 15 and 15 to 0 .

Observation:-

- First the counter goes up from 0 to 15 then the counter goes down from 15 to 0 ..It oscillates like this again and again ..

Arduino Code:-

Timer t;

int p0 = 10;

int p1 = 11, p2 = 12, p3 = 13;

int f = 1;

void setup()

{

Serial.begin(9600);

pinMode(p0, OUTPUT);

pinMode(p1, OUTPUT);

pinMode(p2, OUTPUT);

pinMode(p3, OUTPUT);

digitalWrite(p0, LOW);

digitalWrite(p1, LOW);

digitalWrite(p2, LOW);

digitalWrite(p3, LOW);

osc();

t.every(8010, osc);

}

void loop()

{

t.update();

}

int eventid0, eventid1, eventid2, eventid3;

void stopAllTimers()

{

t.stop(eventid0);

t.stop(eventid1);

t.stop(eventid2);

t.stop(eventid3);

}

void osc()

{

if (f == 0)

{

stopAllTimers();

eventid0 = t.oscillate(p0, 500, HIGH);

eventid1 = t.oscillate(p1, 1000, HIGH);

eventid2 = t.oscillate(p2, 2000, HIGH);

eventid3 = t.oscillate(p3, 4000, HIGH);

f = 1;

}

else if(f == 1)

{

stopAllTimers();

eventid0 = t.oscillate(p0, 500, LOW);

eventid1 = t.oscillate(p1, 1000, LOW);

eventid2 = t.oscillate(p2, 2000, LOW);

eventid3 = t.oscillate(p3, 4000, LOW);

f = 0;

}

}

CONCLUSION:-

The up down desired counter is made using Arduino …

Experiment 6A:-

Objective:- To build a decade counter with a 7 segment display.

Electronic components required:-

- Digital test kit

- Timer (555 IC)

- IC 74HC93 (4 bit binary counter)

- IC CD4511 (BCD to 7 segment decoder)

- 7 Segment display

TinkerCAD screenshot:-

Reference circuits:-

\

Block diagram:-

Procedure:-

- Make appropriate connection to the timer (555 IC) to obtain a time pulse at the output.

- Connect the output of 555 IC to Clock 1 and make connection to clock 0 appropriately in 7493..

- Make the resets 0 and 1 to ground also connect power supply and GND of IC 7493.

- Connect the output pins of 7493 to input pins of BCD convertor 4511 IC.

- Make the common cathode display of 7 segment display and make appropriate of a,b,c,d,e,f,g from 4511 to 7 segment display..

Observation:-

The 7-segment display displays the numbers 0 to 9 then resets to 0 after 6 counts and repeats the process.

Conclusion:-

This is how we can make a 7 segment display that shows number from 0 to 9 …